DORMAKABA PURE® Unframed Glass Partitions – Guide Specification

DORMAKABA Interior Glass Wall Systems bring the benefits of glass to the workplace - transparency, daylighting, versatility, efficiency, space saving, and low cost of ownership. The wall systems come in a range of sleek but rugged minimalist designs and configurations so you can select the system that best matches your project's functional and aesthetic requirements.

DORMAKABA Interior products and solutions help create balance between function and aesthetics in the interior architecture of office units and professional practices. They provide spatial efficiency, transparency, smooth transitions between spaces, comfort and flexibility.

PURE® - Unframed glass partitions with swinging and sliding doors create simple, elegant fronts with extended transparent sightlines. Sliding doors work well wherever space is limited. A sleek top track and a small guide at the sill allow for a smooth transition between office and corridor.

PURE® Enclose - Framed glass partitions with swinging doors provide the aesthetic advantages of glass walls with additional structural and acoustical performance of full framing. The 1-3/4 by 4-1/8 inch metal channels at the floor, ceiling, and walls effectively dampen sound transmission and add structural definition without impeding sightlines. Full framing also accommodates uneven existing floors.

PURE® Sonos – Framed glass wall systems with sliding doors provide the transparency of glass combined with STC rating to enhance privacy.   With an innovative blend of glass and metal, the design fully frames the glass panels, adding rigidity and greater sound protection, without interrupting sight lines.

Fusion - Glass partitions with wood doors constitutes an innovative mixed media concept: cool transparent glass walls merged with warm wood doors. Intriguing options become possible for both modern and traditional décors. Despite the sophisticated combination of elements, the design remains clean and simple - no door control hardware is visible on the door.

We recommend you consult with your DORMAKABA representative, who can be contacted through DORMAKABA USA, Inc.; (800) 523-8483; email: specification@dormakaba-usa.com; website: www.dormakaba.com.

DORMAKABA USA products appear in the following CSI MasterFormat specifications sections:

DORMAKABA Interiors

10 22 15 Fixed Glass Panel Partitions:  PURE®; PURE®  Enclose; Fusion; PURE®  Sonos

10 22 19 Demountable Glass Partitions:  DORMAKABA Uniquin

DORMAKABA GLASS

08 41 26 All-Glass Entrances and Storefronts:  Rail Fittings; Patch Fittings; Point Fittings; Muto XL150; Muto XL120SC; Muto L80; RS120 / RS120 SYNCHRO; DRS120; RSP80; TENSOR

10 22 39 Folding Partitions, Glass:  FSW-G, FSW-C; FSW-ES

10 22 43 Sliding Partitions, Glass:  HSW DRS, HSW-ES, HSW Flex Therm

DORMAKABA ENTRANCE SYSTEMS

08 42 29 Automatic Entrances, Swinging:  ED 50/100/ED250; ED-IG

08 42 29 Automatic Entrances Sliding: ESA 100/ESA 100T; ESA200/ESA 200T; ESA 300/ESA300T;

ESA400/ESA400T; ESA 500

08 42 29 Automatic Sliding Interior Entrances:  MAGNEO

08 42 43 ICU/CCU Entrances:  ICU300 / 300T

CRANE REVOLVING DOOR

08 42 33 Revolving Door Entrances:  1000 Series; 2000 Series; 3000 Series; 4000 Series; KTC Series

This document is Copyright© 2022 by DORMAKABA USA, Inc.

This guide specification is based upon DORMAKABA PURE® - Unframed glass partitions with swinging and sliding doors.

SECTION 10 22 15 – FIXED GLASS PANEL PARTITIONS

GENERAL

      - SUMMARY

        - Section includes fixed, frameless glass panel partitions with [swinging] [and] [sliding] glass doors.

        - Related Requirements:  Related project requirements can be found in the following documents:

          - Section 05 50 00 "Metal Fabrications" for overhead supports that attach glass panel partition tracks to structure.

          - Section 28 13 00 "Access Control" for security access system providing control for door access and intrusion detection systems interfacing with glass panel partition door controls.

      - REFERENCE STANDARDS

Specifier:  If retaining References article, edit the list below to include only those references in the edited section.

        - American Architectural Manufacturers Association (AAMA): www.aama.org:

          - AAMA 611 Voluntary Specification for Anodized Architectural Aluminum

        - American Society of Civil Engineers/Structural Engineering Institute (ASCE/SEI): www.asce.org:

          - ASCE/SEI 7 Minimum Design Loads for Buildings and Other Structure

        - ASTM International (ASTM): www.astm.org:

          - ASTM A666 Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar

          - ASTM B221/ASTM B221M Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes

          - ASTM C1048 Standard Specification for Heat-Strengthened and Fully Tempered Flat Glass

          - ASTM C1172 Standard Specification for Laminated Architectural Flat Glass

          - ASTM E90 Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements

          - ASTM E413 Classification for Rating Sound Insulation

          - ASTM E557 Guide for the Installation of Operable Partitions

        - Builders Hardware Manufacturers Association (BHMA): www.buildershardware.com:

          - ANSI/BHMA A156 Series

        - Code of Federal Regulations

          - 16 CFR 1201 Safety Standard for Architectural Glazing Materials

        - International Code Council (ICC): www.iccsafe.org:

          - ICC A117.1 Accessible and Usable Buildings and Facilities (ANSI)

        - U.S. Architectural & Transportation Barriers Compliance Board:  www.access-board.gov:

          - Americans with Disabilities Act (ADA) and Architectural Barriers Act (ABA) Accessibility Guidelines for Buildings and Facilities

      - ADMINISTRATIVE REQUIREMENTS

        - Coordination:

          - Coordinate installation of glass panel partitions with installation of floor, wall, and ceiling construction to comply with substrate tolerance requirements of partition manufacturer.

          - Coordinate installation of anchors and secondary structural members indicated on approved glass panel partition shop drawings and specified in other sections.

        - Preinstallation Conference:  Conduct conference at Project Site.

      - ACTION SUBMITTALS

        - Product Data:  For each glass panel partition and door component specified, including:

          - Glass panels.

          - Frame and sill tracks.

          - Door hardware and accessories.

        - Shop Drawings: For fixed glass panel partitions.

          - Include plans, elevations, sections, and details. Provide numbered panel installation sequence.

          - Show locations and requirements for tracks, bracing, blocking, and attachments to other work.

        - Samples for Verification:  For each exposed component including hardware, for each color and finish selected, of size indicated below:

          - Glass: Units 12 inches (300 mm) square.

          - Exposed Frame, Track, and Sill Members:  Not less than 6 inches (150 mm) long.

          - Hardware: One of each type of exposed door hardware items.

Specifier:  If required by authorities having jurisdiction, add delegated design requirement for projects requiring seismic bracing designed by Contractor's qualified professional engineer.

      - INFORMATIONAL SUBMITTALS

        - Qualification Data:  For qualified installer.

        - Warranty:  Sample of unexecuted manufacturer warranty.

      - QUALITY ASSURANCE

        - Installer Qualifications:  Experienced Installer equipped and trained for installation of glass panel partitions required for this Project with record of successful completion of not less than five projects of similar scope.

        - Single Source Responsibility:  Provide glass panel partitions and associated hardware by a single manufacturer through a single source.

Specifier: Retain below for large or complex projects where added expense of mockup is justified. Typically treat approved mockup as part of finished work.

        - Mockups:  Provide mockup consisting of initial sections of tracks, frames, and glass panels with operating doors and hardware, in location as directed by Architect. Proceed with work upon approval of mockup by Architect.

      - WARRANTY

Specifier:  Verify that warranty provisions of other products meet those offered by DORMAKABA.

        - Special Manufacturer's Warranty:  Standard form in which manufacturer agrees to repair or replace components of glass panel partitions that demonstrate deterioration or faulty operation due to defects in materials or workmanship under normal use within warranty period specified.

          - Warranty Period:  [Five] years date of Substantial Completion.

- PRODUCTS

      - MANUFACTURERS

Specifier:  Retain option for substitutions below when required for project.

        - Basis-of-Design Product: Provide glass door assemblies manufactured by dormakaba USA, Inc.; (844) 773-2669; email: specifications@dormakaba.com ; website: www.dormakaba.us.

      - PERFORMANCE REQUIREMENTS

Specifier:  If required by authorities having jurisdiction, add delegated design requirement and seismic performance requirements for projects requiring seismic bracing designed by Contractor's qualified professional engineer.

        - Acoustical Performance: Provide glass panel partition tested by qualified testing agency as follows:

          - Sound-Transmission Requirements: Tested for laboratory sound-transmission loss performance according to ASTM E90, determined by ASTM E413, and rated for not less than STC indicated.

      - GLASS PANEL PARTITIONS

        - Fixed Glass Panel Partitions: Frameless glass panel partition with top track and bottom sill guide, with butt-glazed dry joint between panels, and equipped with [swinging] [and] [sliding] doors where indicated.

          - Basis of Design:  dormakaba, PURE.

Specifier:  STC and OITC may vary based upon glass panel selections; consult DORMAKABA representative for details.

          - Sound Transmission Class (STC), ASTM E 90 and Outdoor-Indoor Transmission Class (OITC), ASTM E 1332:

            - Framed partition with 12.0 mm thick laminated glass: STC 33; OITC 30.

            - Swinging door with 12.0 mm thick laminated glass: STC 15; OITC 15.

            - Sliding door with 12.0 mm thick laminated glass: STC 15; OITC 15.

          - Partition Top Track: Aluminum extrusion, low-profile.

          - Sill Guide: Aluminum extrusion.

Specifier:  Select one or more of the glass types from list below as required for project. If more than one glass type is required, retain the drawing designations and indicate locations of each type on Drawings. Consult DORMAKABA representative for availability of additional products.

      - GLASS PANELS AND DOORS

        - Glass Panels, General: Provide glass panels that comply with 16 CFR 1201, Category II requirements for safety glazing. Permanently mark glazing with certification label of the SGCC.

          - Glass and Door Panel Thickness: Thickness required for size of panel based upon manufacturer's written recommendations, but not less than 12 mm.

        - Fully Tempered Clear Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, Quality-Q3; thickness [12.0 mm] [16.0 mm] [19.0 mm].

        - Fully Tempered Ultraclear (Low-Iron) Float Glass: ASTM C 1048, Kind FT, Condition A, Type I, Class 1, Quality-Q3, with visible light transmission of not less than 91 percent; thickness 12.0 mm.

        - Fully Tempered Tinted Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 2, Quality-Q3; thickness 12.0 mm.

        - Laminated Fully Tempered Clear Float Glass: ASTM C1172; consisting of two plies of 6 mm. thick glass with interlayer of 0.060-inch-thick clear polybutyral; unit thickness 12.7 mm.

Specifier:  Consult DORMAKABA representative for more information on the variety of textured and art glass and custom glass treatments that are available.

        - Textured Glass: Clear Fully Tempered Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, Quality-Q3; Finish F1 (textured one side); thickness 12.0 mm.

          - Designer Glass Pattern:  [Skyline Design] [Joel Berman Studios] <Insert designer glass source>, <Insert texture and pattern description>.

      - SWINGING DOORS

        - Accessibility Standard:  Comply with applicable provisions in ADA-ABA Accessibility Guidelines for Buildings and Facilities] [and] [ICC A117.1] [requirements of authorities having jurisdiction].

        - Single Door:  Glass panel matching partition panel material and thickness; [36 by 80 inches (914 by 2032 mm)] [36 by 84 inches (914 by 2134 mm)] <Insert door size>.

        - Double Door:  Glass panel matching partition panel material and thickness; [72 by 80 inches (1829 by 2032 mm)] [72 by 84 inches (1829 by 2134 mm)] <Insert door size>.

      - SLIDING DOORS

Specifier:  Note that if space has 10 or more occupants, sliding doors must be equipped with a breakaway function not available in these units. DORMAKABA sliding interior doors may be used in rooms with occupancy of less than 10, or in spaces with 10 or more occupants room must be equipped with a second means of egress.

        - Accessibility Standard: Comply with applicable provisions in ADA-ABA Accessibility Guidelines for Buildings and Facilities] [and] [ICC A117.1] [requirements of authorities having jurisdiction].

        - Doors: Glass panel matching partition panel material and thickness, of size indicated on Drawings.

        - Sliding Door Track:  Extruded aluminum track designed for operation, size, and weight of glass panel door, with factory-finished head closure trim and seals as required for acoustical performance indicated.

Specifier:  Retain one or more track mounting methods below as required for project. Indicate requirements for secondary structural supports under Section 055000 Metal Fabrications and show details on Drawings.

        - Track Mounting:

          - Ceiling surface-mounted.

          - Ceiling recess-mounted.

          - Partition side-mounted.

        - Door Panel Carriers: Trolley system designed for operation, size, and weight of glass panel door, with ball-bearing wheels.

Specifier:  Select "Manual Sliding Door Operation" or "Automatic Sliding Door Operation" Paragraph below, or retain both if both are required for project.

        - Manual Sliding Door Operation:

Specifier:  Retain single door or synchronized door pair subparagraph below or both.

Option in subparagraphs below describes DORMAKABA's PURE SoftClose option.

          - Single door [with regulated sliding and cushioned close].

            - Basis of Design:  [DORMAKABA PURE] [DORMAKABA PURE with SoftClose].

Specifier: Synchronized door pair operation allows manually pulling on one door leaf to operate both leaves of door pair.

          - Synchronized door pair [with regulated sliding].

            - Basis of Design:  [DORMAKABA PURE] [DORMAKABA PURE with Synchro].

        - Automatic Sliding Door Operation:  Silent operation linear magnetic drive.

          - Basis of Design:  DORMAKABA Magneo.

Specifier:  Select actuator type required for automatic door operation.

          - Actuator:  Push button momentary-contact control switch in 2- by 4- inch junction box.

          - Actuator: Motion sensor: BHMA A156.10, microwave scanner unit, adjustable for bidirectional or unidirectional detection.

      - MATERIALS

        - Aluminum: ASTM B221 (ASTM B221M), with strength and durability characteristics of not less than Alloy 6063-T5.

        - Stainless Steel: ASTM A666, Type 304.

      - FINISHES

        - Aluminum Finish:

          - Clear anodic finish: AAMA 611, AA-M12C22A31, Class II, 0.010 mm or thicker.

          - Powder Coat: Manufacturer's standard thermosetting polyester or acrylic urethane powder coating with cured-film thickness not less than 1.5 mils (0.04 mm), in color selected by Architect from manufacturer's full range.

        - Stainless Steel Finishes:  No. 4 directional satin finish.

Specifier:  DORMAKABA can provide a wide array of applicable door hardware options for glass panel partitions. Consult with DORMAKABA representative for additional options.

For the wide variety of DORMAKABA electronic access control (EAC) options, consult with DORMAKABA representative.

      - DOOR HARDWARE AND FITTINGS

        - Door Hardware, General: All-glass door hardware units in types, sizes, quantities, and mounting locations recommended by manufacturer for glass door types, sizes, and operation. For exposed components, match metal and finish of exposed partition fittings unless otherwise noted.

Specifier:  A wide variety of DORMAKABA decorative operating trim designs are suitable for use with PURE swinging and sliding doors.

Verify compliance of locking ladder pulls with project accessibility requirements, if applicable.

        - Locking Ladder Pull:  Pair of tubular lockable pull handles with thumb turns, Grade 316L stainless steel, accommodating key cylinder, with [floor-recessed deadbolt] [and] [head-mounted deadbolt].

          - Basis of Design:  DORMAKABA, TG 138 Locking Ladder Pulls.

          - Unit Length:  [49 inch (1245 mm)], [60 inch (1524 mm)], [72 inch (1829 mm)], [84 inch (2134 mm)], [__ inch (__ mm)], <insert custom length>.

          - Provide Option for ADA compliance: Staggered at 10” off finished floor on pull side.

        - Pulls and Handles: [One-sided] [Back-to-back].

          - Design: Vertical bar, 1 ¼” diameter, [12 inch] [18 inch] [36 inch] [42 inch] [60 inch].

            - Basis of Design: DORMAKABA, TG 9387 Ladder Pull.

          - Design: Vertical bar, 1 ¼” diameter, [49 inch]  [60 inch] [72 inch] [84 inch].

            - Basis of Design: DORMAKABA, TG138 Non-Locking Ladder Pull.

          - Design: Arced shape matching fitting profile, [13-3/4 inch (350 mm)] [29-1/2 inch (750 mm)].

            - Basis of Design: DORMAKABA, ARCOS Handle Bar.

          - Design: [Specifier insert design] [As selected by Architect from manufacturer's standard designs].

        - Levers:  Wrought, with wrought escutcheons.

          - Design: [Specifier insert design] [As selected by Architect from manufacturer's standard designs].

        - Mechanical Locks and Latches:

          - Mortise Locks:  BHMA A156.13; Grade 1.

            - Basis of Design:  DORMAKABA, M9000 Series.

          - Push-Pull Latches:  BHMA A156.2, Grade 1.

          - Lock and Latch Housings:  Patch mounting to glass panel door, with matching strike mounted in housing on adjacent glass panel.

            - Design: [Specifier insert design] [As selected by Architect from manufacturer's standard designs].

Specifier: Select one of two "Lock Cylinder" paragraphs below.

        - Lock Cylinders: Tumbler type, constructed from brass or bronze, stainless steel, or nickel silver; BHMA A156.5, Grade 1, permanent removable cores; with face finish matching lockset[, keyed to master key system].

        - Lock Cylinders:  As specified in Section 08 71 00 "Door Hardware."

Specifier: Balance of Door Hardware items below are for swinging doors.

        - Electromagnetic Locks: BHMA A156.23; electrically powered; with electromagnet attached to adjacent glass panel and armature plate attached to door; full interior type.

          - Provide power pack for electrified door hardware for concealed installation.

          - Coordinate lock operation with requirements of Section 28 13 00 "Access Control."

        - Patch Fittings for Swinging Doors: At head and sill on pivot side, and at lock and strike at swing side.

          - Basis of Design:  DORMAKABA, PURE Pivot Header.

Specifier:  Typically specify DORMAKABA BTS75 Floor Closer, which is suitable for thin slab applications, and is equipped with a fixed hold-open function; it is suitable for doors weighing up to 260 lb. BTS80 Floor Closer is available for heavy doors and is equipped with an adjustable hold-open function; it is suitable for doors weighing up to 660 lb. Note that BTS80 may not meet ADA-stipulated 5 lb. opening force requirements if applicable.

        - Concealed Floor Closers and Top Pivots: Center hung; BHMA A156.4, Grade 1. Provide housings, bottom insert, top walking beam pivots, mounting plates, and accessories.

          - Basis of Design:  DORMAKABA, [BTS75] [BTS80] Series.

          - Swing: [Single] [Double] acting.

          - Hold Open: [Fixed] [Adjustable].

          - Opening Force: Comply with interior door operating force of authorities having jurisdiction for [accessibility requirements] [and] [egress doors] .

        - Concealed Overhead Closers and Bottom Pivots: Center hung; BHMA A156.4, Grade 1. Provide housings, bottom arms, top walking beam pivots, mounting plates, and accessories.

          - Basis of Design:  DORMAKABA, RTS88

          - Swing: [Single] [Double] acting[, with positive dead stop].

          - Hold Open: [105 degree] [90 degree].

          - Opening Force: Comply with interior door operating force of authorities having jurisdiction for [accessibility requirements] [and] [egress doors] .

Specifier:  Retain "Automatic Door Operators" paragraph if required, and edit options as appropriate for project.

DORMAKABA ED100 Low Energy/Full Energy operator is suitable for doors up to 600 lbs.

        - Automatic Door Operators: Compact electromechanical swing door operator with solid-state controller; BHMA A156.19, power assist, low energy, with opening force required of not more than 5 lbf (22 N); single door, [one-way] [two-way] operation, surface-mounted; size recommended by door operator manufacturer for weight of door.

          - Basis of Design Product:  DORMAKABA, ED100.

          - Activation Device: [Push-plate switch] [Push-button switch] [on each side of door].

          - Cover: [Standard width] [Width of opening].

          - Finish:  Match partition frame finish.

Specifier:  Retain "Exit Device" Paragraph when glass panel partition swinging doors are part of egress from assembly occupancy.

        - Exit Devices: BHMA A156.3 and UL 305.

          - Basis of Design: dormakaba [GP 1000 (Top Latching)] [GP1100 (Bottom Latching)].

          - Tested and approved by BHMA for ANSI 156.3, Grade 1.

          - UL 305 and ULC-S132 tested to the Standard for Panic Hardware.

          - Devices shall be able to be mounted on tempered or tempered laminated glass doors with thicknesses from 3/8 inches through 7/8 inches.

          - Devices shall be capable of being mounted on Metal and Wood Doors with appropriate extension and accessories to accommodate door thicknesses.

          - Function: Device latch to retract when actuating portion of device is depressed with 15 lbs. of pressure.

          - Devices to have “Dogging” feature.  Operation by push-pull when latch toggle is locked down (dogged).

          - Actuation portion of device to have the ability to be adjusted horizontally and vertically +/- ½ inch for precise installation.

          - Latching: Provide << Fixed strike; Electric strike for access control – at top latching devices only>>.

          - Latchbolt to be adjustable.

          - Provide Exterior Pull mounted on pull side; [Insert Pull Type from manufacturer’s standard designs].

          - Pull to have the ability to be adjusted horizontally and vertically +/- ½ inch for precise installation.

          - Provide rim cylinder to accept << electronic; conventional; Large format interchangeable core (LFIC); Small format interchangeable core (SFIC); or>> type core, with <<five-pins; six-pins; seven-pins; or ______>> in compliance with BHMA A156.5 at locations indicated.

          - Coordinate cylinder requirements as required for related sections.

        - Accessory Fittings: [Overhead doorstop] [Floor stops]

- EXECUTION

      - EXAMINATION

        - Examine partition substrates to determine if work is within glass panel partition manufacturer's required tolerances and ready to receive work. Proceed with installation of partitions once conditions affecting installation and performance of partitions meet manufacturer's requirements.

Specifier: If adjacent framed wall construction, including construction above fixed glass panel partitions, is designed to meet acoustical performance criteria, consider retaining paragraph below to ensure that finished construction will meet overall acoustical design objectives.

        - Verify that partition construction adjacent to acoustically-rated glass panel partitions complies with requirements of ASTM E557.

      - PARTITION INSTALLATION

        - General: Comply with glass panel partition manufacturer's written installation instructions and approved shop drawings.

        - Install glass panel partitions after other finishing operations have been completed.

        - Set units level, plumb, and true to line, with uniform joints.

        - Fasten glass panel partition track and sill to building structure and supports as indicated on approved shop drawings, utilizing approved fasteners and spacing.

        - Set, seal, and grout floor closer cases.

      - ADJUSTING

        - Adjust doors and hardware to produce smooth operation and tight, uniform fit.

        - Adjust door closers to required timing and force.

        - Adjust latches and locks for smooth operation.

        - Test and adjust hardware linked to access control system.

        - Replace damaged panels and accessories.

      - CLEANING

        - Clean glass panels in accordance with glass manufacturer's written instructions. Do not use cleaning agents or methods not approved by glass manufacturer.

        - Clean exposed metal surfaces to factory new appearance.

END OF SECTION

Additional Specifiers Notes

Substitution Reviews:  When reviewing substitution requests for other products for compliance with this specification, DORMAKABA recommends particular attention to the following issues:

Compact track, frame, and edge conditions: Design details of DORMAKABA's partitions are what give them the visual appeal that caused them to be selected as a basis of design

SoftClose Sliding Doors: Where selected, this DORMAKABA option has a considerable positive physical and aesthetic impact on the operation of sliding doors

Door Hardware Options: DORMAKABA's position as an international leader in decorative door hardware allows DORMAKABA to offer a complete array of coordinated hardware options and finishes for glass panel partitions.

Coordination:  Make sure you coordinate the following:

Locations and operation of doors including swing or sliding travel

Reflected ceiling plan showing location of full-height partitions in relation to other ceiling-mounted elements

Elevations of partitions indicating details of special decorative glass elements

Glass panel joint locations

Intersections of partitions and other elements of construction

Details indicating acoustical construction continuity in walls and ceiling in rooms formed by partitions; special attention to plenum sound closure, sound attenuating ducts, and floor sound barrier. Refer to ASTM E 557.

Details of support and mounting of track or frame

Details of seismic bracing for partition where required

Details of ceiling finish in relation to track or frame

Details of floor finishes in relation to sill or bottom frame